1. Mounting the 1/2" Air Bulkhead: Drill the Side Wall, Not the Bottom Floor

Do not drill through the flat bottom floor of the bucket for the air line.

- Where to Drill: Drill a clean hole through the curved side wall near the base, about 1.0 to 1.5 inches above the bucket floor (making sure it opens into your 2.5" lower plenum space below the mesh rack).

- Why the Side Wall?

  - Zero Leaks: Putting the air bulkhead through the flat bottom floor makes it sit right where liquid leachate pools, drastically increasing the risk of drips onto your workspace.

  - Easier Connection: Mounting it on the side wall allows your vinyl tubing to exit horizontally and curve smoothly upward into the vertical goose-neck without needing to elevate the bucket on blocks or put awkward 90∘ bends underneath the vessel.

- How to Secure It (No Glue): Thread the 1/2" bulkhead fitting through the drilled hole in the side wall with rubber O-rings/washers on both the inside and outside of the bucket wall. Tighten the locknut firmly to create a watertight mechanical compression seal.

2. Reminding: Mounting the DS18B20 Sensor at the Top

To keep the temperature probe stable and accurately measuring the pile's geometric center over a 14-day run:

- Top Cable Gland Pass-Through: Drill a small hole in the top lid and install a cheap plastic PG7 liquid-tight cable gland (or a split rubber grommet).

- Central Probe Guide Tube: Attach a thin, rigid tube (such as a 1/4" wooden dowel or a piece of rigid plastic tubing) vertically to the underside of the lid extending downward into the compost space.

- Sensor Placement: Zip-tie or tape the DS18B20 probe wire to this guide rod so the metal sensor tip sits precisely in the geometric center of the compost bed (roughly 6 to 8 inches deep). When you snap the lid on, the probe drops into the exact same core spot every time.

3. Vessel Lid: Solid Bucket Lid vs. Styrene Sheets

You must use a real, snap-on plastic bucket lid (equipped with a rubber gasket) rather than cut styrene sheets.

- Thermal Insulation & Air Seal: Thermophilic composting requires trapping latent heat and moisture (52--62∘C). A rigid, gasketed bucket lid seals the top tightly, forcing exhaust steam to exit through your designated vent holes while holding in heat.

- Structural Weakness of Thin Styrene: Cut 0.020" styrene sheets are very thin (roughly the thickness of a heavy cardstock card). Under hot steam and high internal humidity, a flat styrene sheet laid on top will warp, sag, and leak air around the perimeter.

- Proper Use of the Styrene Sheets: Use your styrene sheets inside the vessel (e.g., as internal air distribution plates, splash guards, or baffling) or for small custom mounting brackets—not as the primary structural lid.